Bernd, here there is the numbers for the three systems.

The BLM are analog transmission, so there is no sense for an minimum power definition. For the other, I took the number in the GOH radiation test (http://cms-ecal-optical-links.web.cern.ch/cms-ecal-optical-links/gohirr/GOH_irrad_at_PSI_040227.xls) and the one given by Ben for BIC.

Residual power budget is calculated from the total minus 0.6 dB time 4 connectors and 0.4 dB/km time 3 Km of fiber ~ 4 dB (to be conservative).

For BPM we have only the residual power, the other is estimated.

Cheers

Gianluca

| System | Power (uW) |  |  | Average power | Max Sensitivity | Total Power budget | Residual Power budget (4connection+3 km of fiber) | Wavelength | Max dose rate Gy/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | min | Ave | max | dBm | dBm | dB | dB | nm |  |
| BLM (digital) | 25 | 250 | 500 | -6 | -35 | 29 | 25 | 1310 | 0.004 (average 0.001) estimate for cabel tray on side position 70k(20k) |
| BIC (digital) | 2 | 10 | 40 | -20 | -36 | 16 | 12 | 1310 |  |
| BPM (analog) |  |  |  |  |  |  |  |  |  |