1.25Gbps Pigtailed Coaxial PIN-TIA PD (3.3V)

NTPPT-3EP-x-x

Features

InGaAs long wavelength PIN photodiode with trans-impedance amplifier and decoupling capacitor

High sensitivity with AGC

Single +3.3V power supply

Differential output

Operating temperature from -40 to + 85℃

Single-mode fiber pigtailed with SC,FC connectors

Description

The NTPPT-3 series consist of a reliable InGaAs PIN photodiode with transimpedance amplifier and decoupling capacitor. The NTPPT-3 series are suitable for receiving the light having a wavelength range of 1000 to 1650nm. This photodiode features high quantum efficiency, low dark current and low capacitance for high speed optical fiber communications.

Applications

For telecommunication and data communication systems, from medium to high speed for intra-office, short-haul inter-office and long-haul inter-office applications.

155Mbps, 622Mbps SONET/SDH Receiver

1.25Gbps Gigabit Ethernet Receiver

FTTH, fast Ethernet Receiver

Absolute Maximum Ratings

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc |  |  | 4 | V |  |
| Optical Input Power | Pin |  |  | 2 | dBm |  |
| Operating Temperature | Top | -40 |  | +85 | ℃ |  |
| Storage Temperature | Tstg | -40 |  | +85 | ℃ |  |

Electrical and Optical Characteristics

(Vcc=+3.3V, TC=25℃)

| Parameter | Symbol | Min. | Typ. | Max. | Unit | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc | 3 | 3.3 | 3.6 | V |  |
| Supply Current Note | Icc |  | 30 | 42 | mA |  |
| Detection Range |  | 1.0 |  | 1.65 | ㎛ |  |
| Gain | G | - | 8 | - | V/mW | Differential |
| Bandwidth Note | BW | 830 | - | - | MHz | -3dB |
| Saturation Power | Psat | 0 | - | - | dBm | =1.3㎛ |
| Sensitivity | S | - | -25 | -23 | dBm | BER=10-12 @1.25Gbps |
| Output Resistance | Rout | - | 50 | - |  |  |

Outline Diagram

Horizontal Flange

1

2

3

4

Bottom View

Vertical Flange

1

2

3

4

Bottom View

Pin Description

1: GND(Case)

2: Vcc

3: Vo

1

2

3

4

Bottom View

TIA

1

GND(Case)

0.5nF

PD

Vcc

2

4

Vo

3

Vo

4: Vo

Ordering Information

P:PIN

P: Pigtailed PD

R: Receptacle PD

N:No Connector

1:SC/PC

2:SC/APC

3:FC/PC

4:FC/APC

N: No Flange

V: Vertical

H: Horizontal

T: With TIA

P

NT

P

1:155M, 4:622M, E:1.25G

3: 3.3V, 5: 5V

##### T

P:PD

NT:NEOPTEK