MRV Communications, Inc.

LASERS

PIGTAILED MINI HOUSING

THE Source for your Source

FEATURES	- High Speed

- 1300/1550 NM  Wavelength	- Rear Facet Monitor

- High Optical Power	- Uncooled

- Low Threshold Current	- Mini Housing Package

- High Operating Temperature	- Singlemode / Multimode Fiber Pigtail

CHARACTERISTICS ( T= 25°C )

| Parameter | Sym | Condition | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Wavelength | p | CW | 1270 | 1310 | 1330 | nm |
| Spectral Width |  | CW(RMS) |  | 2 | 5 | nm |
| Threshold Current | Ith | CW |  | 20 | 35 | mA |
| Operating Current | Iop | CW |  | 45 | 60 | mA |
| Output Power | Po | CW | See Below SM(9/125),MM(50/125) |  |  |  |
| Wavelength | p | CW | 1510 | 1550 | 1580 | nm |
| Spectral Width |  | CW(RMS) |  | 3 | 5 | nm |
| Threshold Current | Ith | CW |  | 20 | 35 | mA |
| Operating Current | Iop | CW |  | 45 | 60 | mA |
| Output Power | Po | CW (9/125m) | See Below SM(9/125),MM(50/125) |  |  |  |
| Rise Time/Fall Time | Tr/Tf |  |  | 0.5 |  | ns |
| Forward Voltage | VF | CW |  | 1.2 | 1.7 | V |
| Tracking Error |  | -20 to +60°C |  | ±1 |  | dB |
| Monitor Current | Ipd | CW(Iop) | 0.05 |  |  | mA |
| Monitor Dark Current | Id | Vrd=5V |  | 0.3 | 1.0 | A |
| Monitor Capacitance | Cpd | T=1MHzVrd=5V |  | 10 |  | pF |

OUTPUT POWER ( T= 25°C )

| Part Number 1300 nm Singlemode | Min | Typ | Max | Unit | Part Number 1300 nm Multimode | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MRLDSP001 | 0.1 | 0.2 | 0.4 | mW | MRLDMP005 | 0.5 | 0.7 | 1.0 | mW |
| MRLDSP003 | 0.3 | 0.4 | 0.6 | mW | MRLDMP007 | 0.7 | 1.0 | 1.4 | mW |
| MRLDSP005 | 0.5 | 0.7 | 1.0 | mW | MRLDMP010 | 1.0 | 1.5 | 2.0 | mW |
| MRLDSP010 | 1.0 | 1.2 | 2.0 | mW | MRLDMP015 | 1.5 | 2.0 | 3.0 | mW |
| MRLDSP015 | 1.5 | 1.7 | 3.0 | mW | MRLDMP020 | 2.0 | 3.0 | 4.0 | mW |
| MRLDSP020 | 2.0 | 3.0 | 4.0 | mW | MRLDMP025 | 2.5 | 3.5 | 5.0 | mW |

OUTPUT POWER ( T= 25°C )

| Part Number 1550 nm Singlemode | Min | Typ | Max | Unit | Part Number 1550 nm Multimode | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MRLDSP5001 | 0.1 | 0.2 | 0.4 | mW | MRLDMP5005 | 0.5 | 0.7 | 1.0 | mW |
| MRLDSP5003 | 0.3 | 0.4 | 0.6 | mW | MRLDMP5007 | 0.7 | 1.0 | 1.4 | mW |
| MRLDSP5005 | 0.5 | 0.7 | 1.0 | mW | MRLDMP5010 | 1.0 | 1.2 | 2.0 | mW |
| MRLDSP5010 | 1.0 | 1.2 | 2.0 | mW | MRLDMP5015 | 1.5 | 1.7 | 3.0 | mW |

ABSOLUTE MAXIMUM RATINGS

| PARAMETER | SYMBOL | CONDITIONS | RATING | UNIT |
| --- | --- | --- | --- | --- |
| LD reverse Voltage | VrLD | CW | 2.5 | VOLTS |
| LD Forward Current | If | CW | 150 | mA |
| PD Forward Current | Ifpd | CW | 2.0 | mA |
| PD Reverse Voltage | Vrpd | CW | 15 | V |
| Operating Temp | Top |  | -20 to +60 | °C |
| Storage Temp | Tstg |  | -30 to +80 | °C |

Package Style “SP”

Outline Drawing and Connection (3-Pin)

Outline Drawing and Connection (4-Pin)

input.doc 10/30/98

CAUTION:

Observe Precautions for handling

Electrostatic Sensitive Devices.

Invisible Laser radiation. Avoid Exposure to beam

Class III B Device.