MRV Communications, Inc.

1310 DFB LASERS

14 PIN DUAL INLINE PACKAGE

COOLED

WITH OPTICAL ISOLATOR

THE Source for your Source

FEATURES	- High Speed

- 1300 NM  Wavelength	- Rear Facet Monitor

- High Optical Power	- Thermo-Electric Cooler

- Low Threshold Current	- 14 Pin Dual Inline Package

- With Optical Isolator	- Singlemode Fiber Pigtail

CHARACTERISTICS ( T= 25°C )

| Parameter | Sym | Condition | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Peak Wavelength | p | CW, Po= 1mW | 1290 | 1310 | 1330 | nm |
| Temp. Coeff. of peak wavelength |  | CW, Po= 1mW |  |  | .08 | nm/°C |
| Spectral Width |  | CW, Po= 1mW |  | 0.1 | 0.2 | nm |
| Threshold Current | Ith | CW |  | 20 | 40 | mA |
| Operating Current | Iop | CW |  | 40 | 60 | mA |
| Side Mode Suppression Ratio | Sr | CW, Po= 1mW | 30 | 33 |  | dB |
| Rise Time | Tr |  |  | 0.3 | 0.4 | ns |
| Fall Time | Tf |  |  | 0.4 | 0.5 | ns |
| Forward Voltage | Vf | CW |  | 1.2 | 1.7 | V |
| Thermal Resistance | R | Tth=25°C | 9.0 | 10 | 11 | K |
| Cooler Current | Ite | T=40°C |  | 0.6 | 1.5 | A |
| Cooler Voltage | Vte | T=40°C |  | 1.6 | 3.0 | V |
| Monitor Current | Ipd | CW(Iop) | 0.1 |  |  | mA |
| Monitor Dark Current | Id | Vrd=5V |  | 0.3 | 1.0 | A |
| Monitor Capacitance | Cpd | T=1MHZ Vrd=5V |  | 10 |  | pF |

OUTPUT POWER ( T= 25°C)

| Part Number 1300 nm Singlemode | Min | Typ | Unit |
| --- | --- | --- | --- |
| MRLD14CD005I | 0.5 | 0.75 | mW |
| MRLD14CD0075I | 0.75 | 1.0 | mW |
| MRLD14CD010I | 1.0 | 1.5 | mW |
| MRLD14CD015I | 1.5 | 2.0 | mW |
| MRLD14CD020I | 2.0 | 3.0 | mW |
| MRLD14CD025I | 2.5 | 4.0 | mW |

ABSOLUTE MAXIMUM RATINGS

| PARAMETER | SYMBOL | CONDITIONS | RATING | UNIT |
| --- | --- | --- | --- | --- |
| LD reverse Voltage | VrLD | CW | 2.0 | VOLTS |
| LD Forward Current | If | CW | 150 | mA |
| PD Forward Current | Ifpd | CW | 2.0 | mA |
| PD Reverse Voltage | Vrpd | CW | 15 | V |
| Operating Temp | Top |  | -20 to +60 | °C |
| Storage Temp | Tstg |  | -30 to +80 | °C |

Outline Drawing

Pinout Diagram

| Pin | Function | Pin | Function |
| --- | --- | --- | --- |
| 1 | Cooler Anode | 8 | PD Anode |
| 2 | NC | 9 | Laser Cathode |
| 3 | NC | 10 | Laser Anode and Case Ground |
| 4 | NC | 11 | Thermistor |
| 5 | Laser Anode and Case Ground | 12 | Thermistor |
| 6 | NC | 13 | NC |
| 7 | PD Cathode | 14 | Cooler Cathode |

input.doc 2/4/99