MRV Communications, Inc.

MARVEL SERIES 8E

TRANSMITTER

THE Source for your Source

FEATURES

- 1300 / 1550 nm  Wavelength	- High Speed

- For Singlemode / Multimode Applications	- Low Modal Noise

- High Optical Power	- 8 Pin Package with FC,ST,SC, Pigtail

- Low Operating Current	- High Operating Temperature

Transmitter CHARACTERISTICS ( T= 25°C )

| Parameter | Sym | Conditions | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Wavelength |  | CW | 1260 | 1310 | 1340 | nm |
| Spectral Width |  | CW | 30 |  | 80 | nm |
| Operating Current | Iop | CW |  | 80 | 100 | mA |
| Output Power | Po | S/M CW(9/125 m) |  | SEE CHART BELOW |  |  |
|  |  | M/M CW(50/125 m) |  | SEE CHART BELOW |  |  |
| Wavelength |  | CW | 1510 | 1550 | 1580 | nm |
| SpetCral Width |  | CW | 45 |  | 80 | nm |
| Operating Current | Iop | CW |  | 80 | 100 | mA |
| Output Power | Po | S/M CW(9/125 m) |  | SEE CHART BELOW |  |  |
|  |  | M/M CW(50/125 m) |  | SEE CHART BELOW |  |  |
| Forward Voltage | Vf | CW |  | 1.2 | 1.7 | V |
| Rise/Fall Time | Tr,Tf |  |  | 1.5,2.5 |  | ns |

AVERAGE OUTPUT POWER ( T= 25°C, 50% DUTY CYCLE )

| Part Number 1300 nm Singlemode | Min | Typ | Max | Unit | Pkg ST | Part Number 1300 nm Multimode | Min | Typ | Max | Unit | Pkg ST |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MRTEXX001-8X | 10 | 15 | 20 | W | A,B | MRTEXX003-8X | 30 | 40 | 60 | W | A,B |
| MRTEXX002-8X | 20 | 30 | 40 | W | A | MRTEXX004-8X | 40 | 50 | 80 | W | A |
| MRTEXX003-8X | 30 | 40 | 60 | W | A | MRTEXX005-8X | 50 | 60 | 100 | W | A |

AVERAGE OUTPUT POWER ( T= 25°C, 50% DUTY CYCLE )

| Part Number 1500 nm Singlemode | Min | Typ | Max | Unit | Pkg ST | Part Number 1500nm Multimode | Min | Typ | Max | Unit | ST |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MRTEXX50005-8X | 5 | 7 | 10 | W | A | MRTEXX5002-8X | 20 | 30 | 40 | W | A |
| MRTEXX5001-8X | 10 | 15 | 20 | W | A | MRTEXX5003-8X | 30 | 40 | 60 | W | A |

XX = 	SP- Singlemode Pigtail, MP- Multimode Pigtail, ST- Singlemode ST, MT- Multimode ST

FC- Singlemode FC, MC- Multimode FC. SC-Singlemode SC, MS-Multimode SC

X   =	Pinout Options

Sample Part Number: MRTEST001-82A / B

ABSOLUTE MAXIMUM RATINGS

| PARAMETER | SYMBOL | CONDITION | RATING | UNIT |
| --- | --- | --- | --- | --- |
| Reverse Voltage | Vr | CW | 2.5 | VOLTS |
| Operating Current | Iop | CW | 150 | mA |
| Operating Temp | Top |  | -20 To +70 | °C |
| Storage Temp | Tstg |  | -30 To +80 | °C |

Package Style “ST”/ “MT”-A

Package Style “ST”/ “MT”-B

Package Style “FC”/ “MC”

Package Style “SP”/ “MP”

Package Style “SC”/ “MSC”

Pinout Diagram

input.doc 2/4/98