24.9.2004, E.B.Holzer

BLMs for the Collimator tests in 2004 at the SPS

SPS type BLM chambers:

Chamber diameter, outside:						11.6 cm

Chamber diameter, active:						9.3 cm

Chamber length, outside:						~30 cm

Chamber length, active:						19 cm

Installation of the BLMs for the Collimator test in BA5 SPS

Longitudinal distance from the beginning of the Collimator	 	9.17 m	 (channel 1,2,3,4)

to the beginning of the BLM:						12.5 m (channel 5,6,7,8)

Distance, horizontal, beam line (center) to BLM (center):		+ 11 cm (ch. 3,4,7,8)

- 11 cm (ch. 1,2,5,6)

Distance, vertical beam line (center) to BLM (center):		+ 9.5 cm (ch. 1,3,5,7)

-9.5 cm (ch. 2,4,6,8)

| Channel | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long. from start of coll. to beginning of BLM [m] | 9.17 | 9.17 | 9.17 | 9.17 | 12.5 | 12.5 | 12.5 | 12.5 |
| Hor. center beam line to center BLM [cm] | -11 | -11 | 11 | 11 | -11 | -11 | 11 | 11 |
| Vert. center beam line to center BLM [cm] | 9.5 | -9.5 | 9.5 | -9.5 | 9.5 | -9.5 | 9.5 | -9.5 |

Installation of the BLMs for the Collimator test in TT40

4 standard SPS monitor

at floor level: 1, 2 and 4

on top of the BFCT: 3

distance to the end of the collimator:	channel 1	-347.5 cm	to end of  BLM

channel 2	-234.25 cm	to end of  BLM

channel 3	-148.7 cm	to center of BLM

channel 4	+3 cm		to center of BLM

| Channel | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Long. from end of coll. to end of BLM [m] | -3.475 | -2.3425 |  |  |
| Long. from end of coll. to middle of BLM [m] |  |  | -148.7 | .03 |
| Orientation of BLM | Long. | Long. | Hor. | Hor. |
| Hor. center beam line to center BLM [cm] | +10 ? | +20 ? | 0 | 0 |
| Vert. center beam line to center BLM [cm] | 94 | 94 | 14 | 94 |